ECE 1574 Lab Assignment 4

Embedded System Debugging Techniques

CRNs 11720-11727

Introduction

In this lab, you will focus on some of the debugging techniques used in an embedded system

such as the Fox11 board. If the program is not functioning correctly, you may need to:

- Use printf statements to determine what the program is doing, e.g., using printf statements with ‘if’ statements to determine if the program actually enters the if statement,

- Call the serial_getline() function to wait for user input so that the user can read the computer screen and determine if the program is operating correctly,

- display a variable before you change its value and then display the variable after you changed its value to verify that the variables are the values that you expect them to be.

Although the Buffalo monitor program provides some of these facilities, Buffalo is geared

towards assembly language programming of the 68HC11 processor. For C and C++ programs,

the VTioroutines library is more convenient and useful.

Give some thought to debugging while the program is being written. A program should be

capable of generating a narrative log of what state it is in and what data has been read or

computed. On the Fox11 board, debug output can be written to the HyperTerminal window by

calling the printf() function. Each print should unambiguously identify what the program is

doing and the data value(s) being displayed. Any printf() statement that is no longer

required should not be deleted, and can be commented out by placing // at the start of the line.

LEDs and the LCD display may also be used to indicate program state and the values of

variables.

Debugging is mostly the process of isolating where in the program an error has occurred. Like

Sherlock Holmes, one should first come up with a hypothesis for what has happened, and then

either confirm or refute the hypothesis by looking at the output from debug prints. An example

of a hypothesis is that the wrong branch was taken in an if-else statement. This can be tested

by putting a different printf() message in the two halves of the if statement.

Program Requirements

Implement a program that simulates a simple (modified) electronic combination lock on a door.

The combination value is a sequence of three decimal digits. However, there are three correct

sequences. If the user presses 732 or 759 or 161, the lock should open. Any other sequence will be wrong. The keys from 0 to 9 on the membrane keypad are used as to sequentially enter a

combination value for the door. Although it is possible for a program to detect that a membrane

key has been pressed by calling VTkeypad() in a polling loop, you will instead press and hold

a membrane key and then indicate that the keypad may be read by hitting the Enter key on

your PC’s keyboard. The serial_getline() function can be called to have your program

wait until the Enter key is pressed. Even though no other keyboard characters need to be

entered before hitting the Enter key, a string buffer argument must be supplied to the

serial_getline() function as in Lab 3.

Your program should confirm that a key has been read by lighting up one LED, each time that

the Enter key has been pressed. It should light up one LED for the first digit and the next

LED for the next digit. After hitting the Enter key, the next digit may be selected on the

membrane keypad. These steps will be repeated for each of the three digits in the combination.  Light up the LEDs by starting from the leftmost LED and working your way to the right.  You will only press one key on the membrane keypad at a time.

Before writing your program for this lab, run the program provided in Lab 3 in order to record

the values that are returned by VTkeypad() when each of the digits from 0 to 9 are pressed.

Before doing this, be sure that the keypad has been correctly connected to the Fox11 board. The

values returned are not the digits from 0 to 9 as you might expect, and are instead mask values

where the state of each key is represented by a distinct bit.

At the beginning of your program, you should write the string Enter digits on the top line

of the LCD display. If the correct digits have been entered, display the string Door unlocked

on the bottom line of the LCD display. If the correct digits have not been entered, display the

string Try again on the bottom line of the LCD display and issue a beep. In both cases, the

LEDs should be turned off. As in Lab 3, your main program (excluding the variable declarations

and call to VT_init()) should be contained in an infinite while(1) loop in order to allow

multiple attempts at unlocking the door.  Your program should track the keys that are pressed in the order that they are pressed in.  It is required to use the if-else statements to determine if the user entered the correct combination.  Each digit of the combination that the user enters should have its own variable and at the end of the program is where the if-else statements will be to check the sequence of the keys that were pressed.

In order to help debug your program, display the correct and actual keypad values on the

HyperTerminal window after each digit is entered. Be sure to use a consistent radix

(hexadecimal is recommended) for displaying both values. If the value read from the keypad

does not seem to be what you expect, investigate whether it corresponds to the digit pressed

while the Enter key is pressed, the digit pressed (or absence of a digit pressed) before the

Enter key is pressed, or the digit pressed after the Enter key is pressed.

Program Validation

TA initials:

1)_____

2)_____

3)_____

4)_____

5)_____

(1) The program initially displays Enter digits, and all LEDs are turned

off.

(2) Press and hold the first correct digit on the keypad while hitting the

Enter key. The leftmost LED should light up and a debug message

containing the expected and actual key mask values, in hex, should

appear on the HyperTerminal window.

(3) Press and hold the second correct digit on the keypad while hitting the

Enter key. The second LED should also light up and a debug message

containing the expected and actual key mask values, in hex, should

appear on the HyperTerminal window.

(4) Press and hold the third correct digit on the keypad while hitting the

Enter key. A debug message containing the expected and actual key mask

values, in hex, should appear on the HyperTerminal window. In

addition, the message Door unlocked should appear on the second

line of the LCD display.  All LEDs should be turned off.

(5) Without restarting the program, repeat steps (2) through (5) for an invalid

combination. The results should be the same except that the message

Try again appears on the second line of the LCD display. Also,

repeat with the other two correct sequences which should give the output Door

unlocked.

Once you have completed each of the above steps, please validate your lab by showing the TA

that all five functions work properly. The TA should initial each step above and then sign below

to show that the lab has been validated. This lab (and all further labs) must be validated during

your lab section.

_______________________________

Your Name

____________________                            ___________________/________________________

Date                                                                    GTA name / Signature